CRE52-M (MS-8352E) Firmware Update Procedure

This utility tool applies to Windows OS.

Note: Please install USB driver first if using Windows 98 SE.

Step 1:

Extract the firmware file to destination folder, including OSDWinFM.exe (update utility) and xxx.bin (firmware file).

Step 2

Run OSDWinFM.exe.

Step 3

Click “Browse” to open source firmware file.

Step 4

Click “Upgrade”. Upgrade will start.

NOTE: DO NOT TURN OFF POWER DURING UPGRADE PROCESS.

Improper action may damage the drive.

Step 5

Restart PC when flash procedure completes.