# PIONEER

### DVD-R writer Subsystem  DVR-S201 firmware version explanation list

| Version | Part Number |  | Description |  | Model : Serial Number |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2.02 | DYW1659-C |  |  | ・Target Beta-value of the OPC is changed 0.3 to 0.2 for the PVC 4.7G DVD-R Media. ・Collected return value of the Read Capacity Command with DVD-R ver 2.0 for General Media. The drive has been returning full size of the writable user data area for that media. It is collected to return the size of written user data area. ・Collected the fail of the NWA(Next Writable Address) searching. Time-out error has been occurred sometimes. ・Collected the read error at the first data of the Incomplete Border. When the small data is written from the first address of the Incomplete Border (no closed Border) and the drive try to read this data, it has occurred the read error. |  | DVR-S201/TUCYVK/WL UGTT006570WL~ |
| 2.12 |  | DYW1659-D |  | ・Modified target beta value of OPC to 0.35. ・Modified the returned value of the Read Capacity Command. The Last Recorded Address field was incorrect for DVD-R Version 2.0 for General media. ・Corrected the Next Border Marker address. The Next Border Maker started from 1 ECC block before. ・Modified Border Close sequence. If new border does not create, Last Border is closed with not appendable. ・Applied high frequency modulation of Laser power for writing. ・Supported 4.7G media of Taiyoyuden. |  | DVR-S201/TUCYVK/WL UGTTxxxxxxWL |